画册110-116页

产品标题：

平型帶肋吸盤

Flat ribbed suction cup

主要優勢 Main advantage

.圓形，扁平吸盤，用於處理木質貼面，矽片和太陽能電池；

.處理各種包裝材料，例如 塑料薄膜，紙，泡罩包裝；

.由於扁平的長錐形密封唇和內部支撐，可以輕柔地處理塑料薄膜和紙張。

.Round, flat suction cup for handling wooden veneers, crepe and solar cells;

.Handling various packaging materials such as plastic film, paper, blister

packaging;

.Thanks to the flat long tapered lip and internal support, plastic film and

paper can be handled gently.

平型带肋吸盤-吸盤托架無緩沖型Flat ribbed suction cup-Suction cup bracket with cushion type

| 尺寸圖表 Dimensional chart |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 訂購代號 Order code |  |  |  |  | 參數 Parameter |  |  |  |
| ① | ② | ③ | ④ | ⑤ | A | B | C | 型號 Model |
| 吸盤托架類型 Suction cup bracket type | 吸盤代號 Suction cup code | 吸盤類型 Suction cup type | 材質 Material | 接管方式 Takeover method |  |  |  |  |
| VPA | 18 | BK | S LH | 6J 6B B5 | 18 | 24 | 42 | VPA 18 BK ④⑤ |
|  | 26 |  |  |  | 26 |  |  | VPA 26 BK ④⑤ |
|  | 33 |  |  |  | 33 |  |  | VPA 33 BK ④⑤ |

| 可选配快速接头尺寸圖表 Dimensional chart Optional quick connector |  |  |  |
| --- | --- | --- | --- |
| 快速接头规格 Quick connection specification | 气管直径 Pipe diamete | A | B |
| POC6-M5C | Ø6 | M5 | 14 |
| POC6-01 |  | G1/8 | 12 |
| POC4-M5C | Ø4 | M5 | 14 |

平型带肋吸盤-吸盤托架附緩沖型（直立）Flat ribbed suction cup-Suction cup bracket with cushion type (upright)

| 尺寸圖表 Dimensional chart |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 訂購代號 Order code |  |  |  |  |  |  | 參數 Parameter |  |  |  |  |  |
| ① | ② | ③ | ④ | ⑤ | ⑥ | ⑦ | A | B | C | D | E | 型號 Model |
| 吸盤托架類型 Suction cup bracket type | 吸盤代號 Suction cup code | 緩沖行程 Buffer stroke | 吸盤類型 Suction cup type | 材質 Material | 回轉方式 Swing method | 接管方式 Takeover method |  |  |  |  |  |  |
| VPT | 18 | 10 | BK | S LH | J K | 6J 6B B5 | 18 | 27.5 | 80.5 | 10 | 23 | VPT 18-10 BK ⑤⑥⑦ |
|  |  | 20 |  |  |  |  | 18 |  | 118.5 | 20 | 51 | VPT 18-20 BK ⑤⑥⑦ |
|  |  | 30 |  |  |  |  | 18 |  | 128.5 | 30 | 51 | VPT 18-30 BK ⑤⑥⑦ |
|  |  | 50 |  |  |  |  | 18 |  | 174.5 | 50 | 77 | VPT 18-50 BK ⑤⑥⑦ |
|  | 26 | 10 |  |  |  |  | 26 |  | 80.5 | 10 | 23 | VPT 26-10 BK ⑤⑥⑦ |
|  |  | 20 |  |  |  |  | 26 |  | 118.5 | 20 | 51 | VPT 26-20 BK ⑤⑥⑦ |
|  |  | 30 |  |  |  |  | 26 |  | 128.5 | 30 | 51 | VPT 26-30 BK ⑤⑥⑦ |
|  |  | 50 |  |  |  |  | 26 |  | 174.5 | 50 | 77 | VPT 26-50 BK ⑤⑥⑦ |
|  | 33 | 10 |  |  |  |  | 33 |  | 80.5 | 10 | 23 | VPT 33-10 BK ⑤⑥⑦ |
|  |  | 20 |  |  |  |  | 33 |  | 118.5 | 20 | 51 | VPT 33-20 BK ⑤⑥⑦ |
|  |  | 30 |  |  |  |  | 33 |  | 128.5 | 30 | 51 | VPT 33-30 BK ⑤⑥⑦ |
|  |  | 50 |  |  |  |  | 33 |  | 174.5 | 50 | 77 | VPT 33-50 BK ⑤⑥⑦ |

| 可选配快速接头尺寸圖表 Dimensional chart Optional quick connector |  |  |  |
| --- | --- | --- | --- |
| 快速接头规格 Quick connection specification | 气管直径 Pipe diamete | A | B |
| POC6-M5C | Ø6 | M5 | 14 |
| POC6-01 |  | G1/8 | 12 |
| POC4-M5C | Ø4 | M5 | 14 |

平型带肋吸盤 -附接頭型Flat ribbed suction cup -Attached type

| 尺寸圖表 Dimensional chart |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 訂購代號 Order code |  |  |  | 參數 Parameter |  |  |  |  |
| ① | ② | ③ | ④ | A | B | C | D | 型號 Model |
| 吸盤托架類型 Suction cup bracket type | 吸盤代號 Suction cup code | 材質 Material | 選配接頭 Optional connector |  |  |  |  |  |
| BK | 18 | S LH | M5 | 18 | 24 | 32 | 4.5 | BK 18 ③-M5 |
|  | 26 |  |  | 26 |  |  |  | BK 26 ③-M5 |
|  | 33 |  |  | 33 |  |  |  | BK 33 ③-M5 |

| 尺寸圖表 Dimensional chart |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 訂購代號 Order code |  |  |  | 參數 Parameter |  |  |  |  |
| ① | ② | ③ | ④ | A | B | C | D | 型號 Model |
| 吸盤托架類型 Suction cup bracket type | 吸盤代號 Suction cup code | 材質 Material | 選配接頭 Optional connector |  |  |  |  |  |
| BK | 18 | S LH | 18M | 18 | 24 | 34.5 | 6.5 | BK 18 ③-M5 |
|  | 26 |  |  | 26 |  |  |  | BK 26 ③-M5 |
|  | 33 |  |  | 33 |  |  |  | BK 33 ③-M5 |